## INTEL Processors List

## Desktop Processors:

##### "Clarkdale" (32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX,[citation needed] SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), Intel VT-x

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pentium G6950 |  | 2800 MHz | NA | 533 MHz | 2 x 256 KB | 3 MB | DMI | 21x | 2 x DDR3-1066 |  | 73 W | LGA 1156 | January 2010 |  | $87 |

#### Core i3  "Clarkdale" (32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX,[citation needed] SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), Intel VT-x, Hyper-Threading, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i3-530 |  | 2933 MHz | NA | 733 MHz | 2 x 256 KB | 4 MB | DMI | 22x | 2 x DDR3-1333 |  | 73 W | LGA 1156 | January 2010 |  | $123 |
| Core i3-540 |  | 3067 MHz | NA | 733 MHz | 2 x 256 KB | 4 MB | DMI | 23x | 2 x DDR3-1333 |  | 73 W | LGA 1156 | January 2010 |  | $143 |

#### Core i5

##### "Clarkdale" (32 nm)

- Based on the Intel Westmere microarchitecture

- All models support: MMX,[citation needed] SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT-x, Intel VT-d, Hyper-Threading, Turbo Boost, Smart Cache

- i5-661 will not support Intel VT-d

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i5-650 |  | 3200 MHz | 3.46 GHz | 733 MHz | 2 x 256 KB | 4 MB | DMI | 24x | 2 x DDR3-1333 |  | 73 W | LGA 1156 | January 2010 |  | $176 |
| Core i5-660 |  | 3333 MHz | 3.60 GHz | 733 MHz | 2 x 256 KB | 4 MB | DMI | 25x | 2 x DDR3-1333 |  | 73 W | LGA 1156 | January 2010 |  | $196 |
| Core i5-661 |  | 3333 MHz | 3.60 GHz | 900 MHz | 2 x 256 KB | 4 MB | DMI | 25x | 2 x DDR3-1333 |  | 87 W | LGA 1156 | January 2010 |  | $196 |
| Core i5-670 |  | 3467 MHz | 3.73 GHz | 733 MHz | 2 x 256 KB | 4 MB | DMI | 26x | 2 x DDR3-1333 |  | 73 W | LGA 1156 | January 2010 |  | $284 |

### Quad-Core Desktop Processors

#### Core i5

##### "Lynnfield" (45 nm)

- All models support: MMX,[citation needed] SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT-x, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

- Hyper-Threading is disabled.

| Model Number | sSpec Number | Frequency | Turbo | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i5-750s | SLBLH | 2400 MHz | 2.93 GHz | 4 x 256 KB | 8 MB | DMI | 18x | 2 x DDR3-1333 |  | 82 W | LGA 1156 | Q1 2010 | BX80605I5750s | $259 |

#### Core i7

##### "Lynnfield" (45 nm)

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT-x, Intel VT-d, Hyper-Threading, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i7-860s | SLBLG | 2533 MHz | 3.20 GHz | 4 x 256 KB | 8 MB | DMI | 19x | 2 x DDR3-1333 |  | 82 W | LGA 1156 | Q1 2010 | BX80605I7860s | $337 |

### Six-Core Desktop Processors

#### Core i9

##### "Gulftown" (32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

| Model Number | sSpec Number | Frequency | Turbo | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i9-xxx XE |  | 3333 MHz | 3.?? GHz | 6 x 256 KB | 12 MB | 2 x 6.4 GT/s QPI | 25x | 3 x DDR3-1066 | ? V | 95 W | LGA 1366 | Q1 2010 |  | $999 |

## Server Processors

### Xeon DP, Six-Core

#### "Gulftown" (32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- All models support dual-processor configurations

| Model Number | sSpec Number | Frequency | Turbo | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xeon ?56xx |  | 2400 MHz |  | 6 x 256 KB | 12 MB | 2 x ??? GT/s QPI | 18x | 3 x DDR3-1333 | 1.055 V |  | LGA 1366 | Q1 2010 |  | $999 |

### Xeon MP, Eight-Core

#### "Beckton" (45 nm)

- Based on Intel Nehalem microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- All models support quad-processor configurations

| Model Number | sSpec Number | Frequency | Turbo | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Xeon ?75xx |  |  |  | 8 x 256 KB | 24 MB | 4 x 6.4 GT/s QPI |  | 4 x DDR3-1333 |  |  | LGA 1567 | Q1 2010 |  |  |

## Mobile Processors

### Dual-Core Mobile Processors

#### Core i5

##### "Arrandale" (standard voltage; 32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i5-520M |  | 2400 MHz | 2.93 GHz | 766 MHz | 2 x 256 KB | 3 MB | DMI | 18x | 2 x DDR3-1066 |  | 35 W | µPGA-989 | Q4 2009 |  | $225 |
| Core i5-540M |  | 2533 MHz | 3.06 GHz | 766 MHz | 2 x 256 KB | 3 MB | DMI | 19x | 2 x DDR3-1066 |  | 35 W | µPGA-989 | Q4 2009 |  | $257 |

#### Core i7

##### "Arrandale" (ultra-low voltage; 32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i7-620UM |  | 1067 MHz | 2.13 GHz | 500 MHz | 2 x 256 KB | 4 MB | DMI | 8x | 2 x DDR3-1066 |  | 18 W | µPGA-989 | Q4 2009 |  | $278 |
| Core i7-640UM |  | 1200 MHz | 2.26 GHz | 500 MHz | 2 x 256 KB | 4 MB | DMI | 9x | 2 x DDR3-1066 |  | 18 W | µPGA-989 | Q4 2009 |  | $305 |

##### "Arrandale" (low voltage; 32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i7-620LM |  | 2000 MHz | 2.80 GHz | 533 MHz | 2 x 256 KB | 4 MB | DMI | 15x | 2 x DDR3-1066 |  | 25 W | µPGA-989 | Q4 2009 |  | $305 |
| Core i7-640LM |  | 2133 MHz | 2.93 GHz | 533 MHz | 2 x 256 KB | 4 MB | DMI | 16x | 2 x DDR3-1066 |  | 25 W | µPGA-989 | Q4 2009 |  | $332 |

##### "Arrandale" (standard voltage; 32 nm)

- Based on Intel Westmere microarchitecture

- All models support: MMX, SSE, SSE2, SSE3, SSSE3, SSE4.1, SSE4.2, Enhanced Intel SpeedStep Technology (EIST), Intel 64, XD bit (an NX bit implementation), TXT, Intel VT, Hyper-Threading, Turbo Boost, Smart Cache

- FSB will be replaced with DMI.

| Model Number | sSpec Number | Frequency | Turbo | GPU Frequency | L2 Cache | L3 Cache | I/O Bus | Mult | Memory | Voltage | TDP | Socket | Release Date | Part Number(s) | Release Price (USD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Core i7-620M |  | 2667 MHz | 3.33 GHz | 766 MHz | 2 x 256 KB | 4 MB | DMI | 20x | 2 x DDR3-1066 |  | 35 W | µPGA-989 | Q4 2009 |  | $332 |